# SCOUT-FL — Research Plan for IEEE TWC

Sensing-Coverage and Uncertainty-Aware Federated Client Selection for ISAC Networks

Status: research planning phase (no writing/coding yet). All papers in §A were verified via live web search this session unless flagged. Theory in §G cross-checked against canonical sources.

# Context (why this project, and the honest framing)

ISAC is a confirmed 6G pillar and FL is already used in ISAC/ISCC (OTA aggregation, beamforming, resource allocation, RIS, edge perception). The goal is a TWC-grade paper on *sensing-aware client selection* for FL-enabled ISAC. The opportunity is real but narrower and more contested than the brief assumed: at least three TWC / TWC-2024–26 papers already do "sensing-aware FL device scheduling / resource allocation." The chief threat is Asaad–Wang–Tabassum, IEEE TWC 2025, which already performs sensing-aware OTA-FEEL device scheduling + beamforming using a combined aggregation-MSE + CRB metric.

Therefore the project only works if SCOUT-FL attacks a wedge none of them occupy (developed in §C). The plan below is built around that wedge and is engineered to survive the "this is incremental over Asaad" reviewer attack.

User decisions locked in: scenario = multi-target + region coverage map; evaluation = synthetic primary + one semi-real dataset. For the algorithm and the sensing metric, the user asked to plan all options with priority scores — see §F and §F-bis.

# 0. Headline strategic finding

Do NOT claim "FL not applied to ISAC" (false), nor "sensing quality affecting FL convergence is new" (Wen et al. TWC-2026 already couples sensing noise → data quality → convergence), nor "sensing-aware device scheduling for OTA-FEEL is new" (Asaad et al. TWC-2025).

The defensible novelty = treat the sensing task as a co-equal, persistent mission whose *informational value is set-dependent and geometry-dependent*, and co-schedule it with learning:

Multi-target + spatial coverage (Asaad = single target response; SCOUT = many targets/regions).

Submodular log-det-FIM viewpoint diversity — redundant high-SNR clients add little; complementary-angle clients add a lot. (No ISAC-FL selector uses this.)

Dynamic sensing-coverage uncertainty map with freshness/AoI over regions (perception maintenance), feeding selection.

Fairness over clients AND regions as a first-class submodular term.

Spatially non-IID *sensing* data from viewpoints linked into the convergence analysis.

A unified monotone-submodular utility with provable greedy guarantees under wireless/energy constraints.

Theory nuance that drives the whole formulation: log-det FIM (D-optimal) is *provably monotone submodular* → greedy (1−1/e) [Shamaiah-Banerjee-Vikalo CDC'10; Nemhauser-Wolsey-Fisher'78]. CRB / trace-of-inverse-FIM (A-optimal, which is what "inverse CRB" really is) is only weakly submodular — no clean guarantee. ⇒ make log-det-FIM the *objective*; make CRB a *constraint + evaluation metric*.

# A. Literature Map (verified this session)

✓ = title/venue verified via web this session. (•) = standard/well-known method, not ISAC-FL. (?) = surfaced by sub-agent, not independently verified — screen before citing.

| # | Paper (short) | Venue / Year | Problem | Sensing metric | FL / aggregation | Selection / scheduling | What it MISSES vs SCOUT-FL | Relevance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ✓1 | OTA-FL Client Selection in ISCC | IEEE ICC-W 2024, pp.804–809 (doc 10615482; ZJU+RWTH) | Client selection + power control for AirComp-FL | Target detection as a constraint | AirComp OTA-FL | Selects clients + power s.t. detection requirement | Sensing is a *constraint*, not a learning-relevant *utility*; no diversity/coverage/fairness/FIM; single workshop scope | Closest on "client selection" label — but shallow; cite as the thing we deepen |
| ✓2 | OTA-FEEL w/ Integrated Sensing: Joint Scheduling & Beamforming (Asaad, Wang, Tabassum) | IEEE TWC 2025, 24(4):3273–3288 (arXiv 2501.06334) | Robust OTA-FEEL where PS senses to cancel target-echo interference on analog aggregation | CRB of a *single* target response | OTA-FEEL; derives global-model MSE | MINLP; "step-wise omit least-effective devices" via combined MSE+CRB metric; AO downlink-BF + uplink post-processing; convergence+complexity given | Single target; no spatial diversity, no FIM/log-det selection, no coverage map, no freshness, no fairness, no non-IID-sensing | PRIMARY COMPETITOR. Must differentiate sharply + reproduce as baseline |
| ✓3 | ISCC for OTA-FEEL (Wen, Xie, Cao, Cui, Xu, Shi, Cui) | IEEE TWC 2026 (arXiv 2508.15185) | Convergence under sensing noise + OTA distortion; resource allocation | Sensing noise → training-data quality | OTA-FEEL; convergence analysis | Resource allocation (scheduling/power/compute) | No selection-utility, no diversity/coverage/fairness, single-objective | Owns "sensing-quality→convergence" — we must cite, not re-claim |
| ✓4 | ISCC for OTA-FL in 6G (Du et al.) | IEEE IoT-J 2024, 11(21):35551–35567 | BS jointly serves sense+comm+comp + OTA-FL | Sensing service / SNR | OTA-FL | Joint resource allocation | No sensing-aware *selection*, no diversity/coverage/fairness | Foundational ISCC-FL framing |
| ✓5 | Comm-Efficient FL for Multiagent ISAC (Ouyang et al.) | IEEE Systems J. 2024 | Non-IID in large-scale multiagent ISAC FL | — | FL; comm-efficient | GAN+DQN data augmentation for non-IID (DAOA) | Not selection; complementary tool for our non-IID story | Cite for non-IID handling |
| ✓6 | Vertical FEEL in Edge Perception (Cao, Wen, Bi, Cui, Zhu, Hu, Eldar) | IEEE TMC 2026 (arXiv 2512.03374) | Multi-view sensing, feature-partitioned VFEEL, OTA feature aggregation | Multi-view sensing features | Vertical FEEL; OTA feature agg | ISCC resource allocation | Vertical (feature) FL, not horizontal client *selection*; no submodular diversity/coverage | Multi-view precedent — supports our "viewpoint diversity" premise |
| ✓7 | ISCC for UAV-Assisted FEEL | IEEE TWC 2024 (arXiv 2306.02990; doc 10829532) | Joint UAV deployment + resource alloc; HAR via wireless sensing | Wireless-sensing data acquisition | FEEL | UAV placement + resource allocation | Placement not client *selection*; no FIM diversity/coverage/fairness | Strong neighbor; another "sensing-aware FEEL" precedent |
| ✓8 | Empowering ISAC w/ FL (Satellite + RIS-STIN) | IEEE TWC 2024 (doc 10768935; DOI 10.1109/TWC.2024.3502394) | Resource allocation in RIS-STIN-ISAC | ISAC SE | Federated DRL (F-DDPG) | DRL resource allocation | Not selection; DRL not submodular | Shows DRL-for-ISAC-FL is taken turf (informs our A3 option) |
| (•) | DivFL (Balakrishnan et al.) | ICLR 2022 | Diverse FL client selection | — (gradient space) | FedAvg | Submodular facility-location over gradients; (1−1/e) | No sensing at all | Methodological backbone for f_learn |
| (•) | Oort (Lai et al.) | OSDI 2021 | Utility-based FL client selection | — | FedAvg | Statistical+system utility | No sensing | Baseline + utility-design precedent |
| (•) | Power-of-choice (Cho et al.) | AISTATS 2022 | Biased client selection | — | FedAvg | Loss-based sampling | No sensing | Baseline |
| (•) | Shamaiah-Banerjee-Vikalo | IEEE CDC 2010 | Greedy sensor selection | log-det FIM submodular | — | Greedy (1−1/e) | — | Core theorem for f_sense |
| (•) | Joshi & Boyd | IEEE T-SP 2009 | Sensor selection via convex opt | det(FIM) / CRB | — | Convex relaxation | — | Convex-relaxation baseline for selection |
| (•) | Krause-Singh-Guestrin | JMLR 2008 | Near-optimal sensor placement | Mutual info (submodular) | — | Greedy | — | Submodular-sensing precedent |
| (•) | Nemhauser-Wolsey-Fisher | Math. Prog. 1978 | Submodular max | — | — | Greedy (1−1/e), tight | — | Approximation-guarantee root |
| (•) | Harshaw et al. | ICML 2019 | Submodular minus modular (distorted greedy) | — | — | (1−1/e)g(S)−c(S) | — | How to put energy/latency cost in the objective |
| (?) | AoI-based Client Selection (FedAoI) | "IEEE TMC 2024" (?) | Freshness-aware selection | — (update AoI) | FedAvg | AoI selection | Update-AoI ≠ sensing-coverage-AoI | Verify; supports our freshness term if real |

Additional ISCC-FL works to screen (found, not deep-read): arXiv 2409.11240 (FL-ISCC frameworks/analysis), arXiv 2504.01443 (Split-FL UAV ISCC), arXiv 2504.18143 (Full-duplex ISCC low-altitude), "Federated Edge Learning via ISCC" (ResearchGate). Newly surfaced (screen before drafting Related Work): arXiv 2603.15783 (multi-objective OTA-FEEL + *collaborative ISAC* — possible direct competitor), arXiv 2604.26741 (signal-level ISCC optimal power control — ICC-W'24 follow-on), arXiv 2212.00491 (gradient+channel-aware OTA scheduling — adopt as a scheduling baseline). Note: Paper ✓3 (Wen) is now IEEE-published — IEEE Xplore doc 11143883.

# B. Gap Analysis (precise)

What exists: (i) sensing-aware OTA-FEEL *device scheduling* where sensing protects aggregation (Asaad TWC-25); (ii) sensing-noise → convergence coupling + resource allocation (Wen TWC-26); (iii) ISCC resource allocation (Du IoT-J-24, UAV-FEEL TWC-24); (iv) FL client *selection* with detection as a *constraint* (ICC-W-24); (v) non-IID via augmentation (Ouyang-24); (vi) multi-view *vertical* FEEL (Cao TMC-26); (vii) DRL resource allocation for ISAC-FL (TWC-24). Submodular *diverse* client selection exists but only in gradient space (DivFL), with no sensing.

What is unsolved (the gap SCOUT-FL fills): No work selects FL clients by their marginal informational contribution to a *multi-target / spatial-coverage* sensing mission — i.e., none models that the *same selection decision* jointly determines (a) FL model quality and (b) set-dependent, geometry-dependent sensing information where redundant viewpoints are nearly worthless and complementary viewpoints are valuable (submodular log-det FIM), under (c) a dynamic region-coverage/freshness map, with (d) client- and region-level fairness, and (e) spatially non-IID sensing data entering the convergence bound — all with provable approximation guarantees under OTA/energy/latency constraints.

One-line gap statement (paper-ready draft): *"Existing FL-enabled ISAC scheduling treats sensing as a single-target QoS constraint, an aggregation-protection signal, or a data-quality noise term; none formulates client selection as joint maximization of FL learning utility and set-dependent multi-view sensing information over a dynamically aging spatial coverage map, with fairness and provable guarantees."*

# C. Novelty Refinement (critical — read before committing)

Verdict: novelty is REAL but THIN at the edges; it must be defended, not assumed. Honest assessment:

❌ "Sensing-aware FL scheduling" alone → dead on arrival (Asaad).

❌ "Sensing affects convergence" → taken (Wen).

⚠️ "FIM/CRB for selection" → exists in *sensor selection*, not in *FL client selection* — novel only as the bridge, and only if non-trivially coupled to learning.

✅ The combination is open and the *spatial multi-view + dynamic coverage + fairness + submodular guarantee in ISAC-FL* has no single precedent.

Strengthen novelty with these (in priority order):

Reframe the mission, not just the metric: SCOUT-FL is *perception-maintenance-aware FL* — the network must keep an up-to-date map of a region while training a model, and client selection serves both. This framing is fresh and quotable.

Prove the "high-SNR ≠ high-value" theorem empirically + analytically: construct a case where a medium-SNR complementary-angle client beats two high-SNR co-located clients on localization CRB. This is the paper's "aha" and the cleanest differentiator from any SNR/CRB-threshold scheme.

Make the coverage map dynamic and couple it to mobility (regions age; targets move) → an *online* selection problem (not one-shot). This is where the bandit/DRL variant (§F-A3) earns its place and adds a regret/Lyapunov result no competitor has.

Couple spatially-non-IID *sensing-derived* data to the convergence bound so the sensing term is *learning-relevant*, not bolted on. This answers "why is this a wireless/learning contribution and not a sensor-selection paper."

If a reviewer finds a paper doing exactly (1)+(2)+(3): fallback wedges = (a) multi-cell / cell-free ISAC-FL selection (most competitors are single-cell), (b) provable fairness over *regions* (spatial fairness, not client fairness), (c) the online regret guarantee under unknown FIM.

# D. TWC Suitability Plan (make it a *wireless* paper, not an FL paper)

TWC rewards rigorous wireless modeling + optimization + theory + extensive realistic simulation. To pass:

Lead with the wireless system model (ISAC signal model, channel, AirComp aggregation, CRB derivation) before any ML.

Sensing math must be first-class: derive the per-client per-target FIM from the ISAC observation model (range/angle estimation), and the AirComp aggregation MSE in closed form. Borrow rigor level from Asaad TWC-25.

The optimization must look like a TWC problem: a constrained MINLP-flavored selection + resource-allocation problem, with constraints on CRB, aggregation MSE, transmit power, bandwidth, latency, energy, and a cardinality/matroid budget.

Include a convergence theorem linking selection → expected per-round descent (ties learning to the wireless decision).

Add a resource-allocation layer (power/bandwidth, optionally receive beamforming/equalizer) so it isn't "just submodular selection."

Simulate under realistic wireless conditions: Rician/Rayleigh fading, path loss, imperfect CSI, mobility, RCS variation, multiple targets, client dropout.

Keep ML deliberately simple (one CNN/MLP, standard datasets) so reviewers don't read it as an ML paper.

# E. Mathematical Model (proposed; scenario = multi-target + region coverage map)

Network. One ISAC PS/BS with N_t Tx / N_r Rx antennas (multi-cell as extension); K edge clients. Environment has M discrete targets and is partitioned into R regions {R_r}. Round t: PS picks S_t⊆[K], |S_t|≤ S_{max}.

Sensing model (per client k, target m). ISAC echo → estimate target params \boldsymbolθ_m=(range,angle). Per-client Fisher information matrix J_{k,m}(\boldsymbolθ_m)⪰ 0, scaling with sensing SNR γ_{k,m} (path loss, RCS, beamforming gain) and viewing geometry (angle) — so J_{k,m} is *directional* and complementary angles give complementary information. Accumulated FIM under S: J_m(S)=J_0+Σ_{k∈S}J_{k,m}. Localization CRB: CRB_m(S)=tr J_m(S)^{-1} (A-optimal). D-optimal info: logdetJ_m(S).

Sensing-diversity utility (centerpiece, submodular):

f_{sense}(S)=Σ_{m}w_m\big[logdet\big(J_0+\textstyleΣ_{k∈S}J_{k,m}\big)-logdetJ_0\big].

Coverage / freshness map (dynamic):

U_r(t{+}1)=ρ_r U_r(t)+ξ_r(t)-Σ_{k∈S_t}c_{k,r}(t), with persistence ρ_r, mobility/innovation ξ_r, contribution c_{k,r}≥0.

Coverage utility: f_{cov}(S;t)=Σ_r U_r(t) g\big(\textstyleΣ_{k∈S}c_{k,r}\big), g concave saturating (e.g. 1-e^{-·} or min(·,U_r)) ⇒ submodular; unsensed regions accumulate U_r → get prioritized (freshness/AoI built in).

Communication / aggregation (OTA / AirComp). Uplink analog superposition: y=Σ_{k∈S}\sqrt{p_k} h_k b_k x_k+n, receive equalizer r. Aggregation MSE MSE_{agg}(S,{p_k},r) in closed form (standard AirComp). Digital-variant rate/latency available; primary = analog OTA per the brief.

FL model. F(w)=Σ_k\frac{n_k}{n}F_k(w); spatially non-IID because client data come from different viewpoints/regions. Partial participation S_t; OTA distortion injects noise into the update.

Learning utility (submodular): facility-location over gradients/representations (DivFL-style): f_{learn}(S)=Σ_{j∈[K]}max_{k∈S} sim(∇ F_j,∇ F_k) (or loss/grad-norm-weighted coverage).

Fairness utility (submodular): concave saturating coverage of under-selected clients/regions, f_{fair}(S;t)=Σ_k φ(a_k(t)) 1[k∈S] with φ increasing in age-since-selection a_k, plus a region-fairness analogue.

Per-round optimization.

max_{S_t, {p_k}, r}   α f_{learn}+λ_s f_{sense}+λ_c f_{cov}+λ_f f_{fair}

s.t.  |S_t|≤ S_{max};  MSE_{agg}≤ε;  CRB_m(S_t)≤\Gamma_m \forall m;  latency≤ T_{max};  energy≤ E_{max};  Σ_k p_k≤ P,  bw budget.

Formulation choice (λ strategy — PRIMARY, resolves the "hand-tuned weights" risk): prefer the constrained formulation above — sensing (CRB), aggregation MSE, latency, energy, power are hard constraints, so the objective retains only learning + sensing-information (and optionally coverage/fairness) as a monotone submodular utility. This is cleaner for TWC and avoids arbitrary weighted-sum tuning. Drive the number of free weights toward zero (ideally only the learning↔sensing-info trade, or none if sensing-info is also a constraint). For any weight that remains: normalize each utility to [0,1] and report a sensitivity sweep + Pareto front, fixing the default operating point by a stated rule (e.g., the Pareto knee), never by cherry-picking.

Cost handling: with energy/latency as constraints the objective is monotone submodular ⇒ greedy guarantees hold (cardinality → (1−1/e); knapsack → cost-benefit greedy / Sviridenko). If instead costs are folded into the objective (utility − energy − latency), use distorted greedy (Harshaw'19): f(S)≥(1-1/e) g(S)-c(S).

Notation/units to fix early: angles in rad, CRB in m²/rad², energy in J, latency in s; normalize any surviving soft weights and run a unit/scale audit before tuning.

# F. Algorithm Design — three options, priority-scored (answers Q1)

Recommended composition: A1 as the core contribution; A3 as a second (online) contribution that adds a regret/Lyapunov result and handles unknown FIM/mobility; A2 as an optional resource-allocation subsection (not full MINLP) to look "wireless." This gives a two-result TWC paper.

| Opt | Approach | Priority | What it buys | Cost / risk |
| --- | --- | --- | --- | --- |
| A1 | Submodular greedy selection (log-det-FIM) + closed-form/convex power-bandwidth layer. Lazy-greedy (CELF) per round under cardinality+knapsack; allocate resources for the chosen set. | 9/10 | Clean (1−1/e); strong theory; differentiates on multi-view diversity; cheap to run. | Could read as "just greedy" → mitigate with the resource layer + convergence theorem + the high-SNR≠value result. |
| A2 | Joint selection + receive beamforming/equalizer + power via alternating optimization (reduced MINLP, not full Tx-BF). | 6/10 as an *extension* (5/10 as standalone primary) | Maximal "wireless" credibility; competes on Asaad's turf. | Highest overlap risk with TWC-25; heavy; diminishing novelty. Do a *reduced* AO subsection only. |
| A3 | Online learning-based selection — contextual bandit (LinUCB/Thompson) or Lyapunov-drift-plus-penalty when FIM/channels are unknown & regions/targets move; regret or drift bound. | 7/10 as a *second contribution* (4/10 as primary) | Handles mobility, unknown sensing quality, long-term fairness via virtual queues; adds a theorem competitors lack. | Weak worst-case theory if used alone; more sim work. Pairs *well* with A1 (offline-greedy oracle vs online-learned). |

Algorithm sketch (SCOUT-FL, A1+A3 hybrid): each round, observe channel/sensing context + coverage map {U_r} + fairness ages; run lazy-greedy marginal-gain selection of S_t under constraints (offline-info oracle = A1); when FIM/channels unknown, replace marginal-gain estimates with upper-confidence / posterior estimates and carry virtual queues for fairness (online = A3); allocate power/bandwidth (closed-form MMSE-type for AirComp); aggregate via AirComp; update U_r and ages. Complexity O(S_{max} K κ) with lazy eval (κ = marginal-gain cost, dominated by a rank-1 log-det update O(d^2) per target).

# F-bis. Sensing-metric options — priority-scored (answers Q2)

Recommended composition: optimize M1, constrain + evaluate with M2, demonstrate M3 in a detection use-case. They coexist; you are *not* forced to pick one.

| Opt | Metric | Priority | Theory status | Role in paper |
| --- | --- | --- | --- | --- |
| M1 | log-det FIM (D-optimal) | 9/10 | Provably monotone submodular → greedy (1−1/e) [Shamaiah'10; Nemhauser'78]. Unconditional for PSD J_k. | Objective / centerpiece. Carries the main theorem. |
| M2 | CRB / trace-inverse-FIM (A-optimal) (= the brief's "inverse CRB") | 7/10 | Weakly submodular only; greedy gets (1-e^{-γ}) via submodularity ratio γ; near-submodular only in low-SNR regime [Chamon-Ribeiro'17; Bian'17]. | Constraint (CRB_m≤\Gamma_m) + headline evaluation metric (localization RMSE). Optional secondary proposition with γ-bound. |
| M3 | Detection probability / mutual information | 5/10 | MI submodular for Gaussian (Krause'08); detection-prob generally not. | Use-case / extra evaluation (detection-prob plots), a scenario variant. Not the optimizer's objective. |

# G. Theory (target lemmas/propositions — realistic to prove)

MINIMUM THEORY FLOOR (non-negotiable — without it the paper is "just a new scoring function for client selection," too weak for TWC): (1) log-det FIM utility is monotone submodular [P1]; (2) greedy selection has an approximation guarantee [P4]; (3) AirComp aggregation MSE provably affects FL convergence [P6]; (4) sensing quality / noise / selection affects expected per-round learning improvement [P6]; (5) complexity analysis [item 8]. The convergence proof (3–4) may be *moderate* (PL / non-convex template from Asaad'25, Wen'26), but it must exist.

P1 (sensing submodularity): f_{sense} is monotone non-decreasing & submodular for PSD J_{k,m} (log-det / matrix-concavity; sum preserves). *Solid.*

P2 (coverage submodularity): f_{cov} monotone submodular for concave saturating g; map U_r(t) bounded under ρ_r<1. *Solid.*

P3 (learning submodularity): f_{learn} (facility location) submodular [DivFL]. *Solid (cite).*

P4 (composite guarantee): nonneg weighted sum of monotone submodular ⇒ monotone submodular ⇒ greedy (1-1/e) under cardinality; Sviridenko/cost-benefit greedy (1-1/e) under knapsack; distorted greedy (1-1/e)g-c if costs in objective [Harshaw'19]. *Solid.*

P5 (CRB corollary / weak-submodular variant): (1-e^{-γ}) guarantee for the A-optimal/CRB objective via submodularity ratio γ; characterize when γ→1 (low-SNR). *Moderate.*

P6 (convergence): under OTA distortion + partial participation + spatially-non-IID sensing data, expected per-round descent is lower-bounded by a function of the selected utility U(S_t), with an explicit aggregation-MSE penalty term; hence maximizing U accelerates convergence. Derive in the L-smooth / bounded-variance / PL or non-convex setting (template from Asaad TWC-25, Wen TWC-26, OTA-FEEL analyses). *Hardest but essential — this makes sensing learning-relevant.*

P7 (online regret / drift, for A3): sublinear regret (bandit) or drift-plus-penalty bound with long-term fairness feasibility (virtual queues). *Moderate; only if A3 included.*

Complexity: lazy-greedy cost, rank-1 log-det updates, total per-round cost; compare to convex-relaxation (Joshi-Boyd) and exhaustive.

# H. Experiment Plan

Testbeds: (1) Synthetic ISAC simulator (primary) — K clients, M moving targets, R regions, Rician/Rayleigh fading, path loss, RCS variation, beamforming gains, AirComp aggregation, imperfect CSI, mobility, dropout; full control of non-IID severity, target/region density, budgets. (2) One semi-real dataset for the learning task mapped to spatially non-IID clients — candidates: DeepSense 6G (multimodal, real), WiMANS (Wi-Fi multi-user CSI), or Wi-Fi-CSI HAR / radar HAR (UoG). Pick one; map samples to viewpoints/regions to realize spatial non-IID.

Baselines: random; full-participation FedAvg (if feasible); loss-based (power-of-choice); grad-norm; channel/SNR-aware; Oort; DivFL (diversity-only, no sensing); sensing-SNR-only; CRB-only; fairness-only; reproduced Asaad-style MSE+CRB scheduling; reproduced ICC-W-24 detection-constrained selection; convex-relaxation selection (Joshi-Boyd); SCOUT-FL (full).

Ablations: SCOUT-FL without {sensing term / coverage term / fairness term / aggregation-MSE term / online module}; D-optimal vs A-optimal objective; greedy vs distorted-greedy vs convex-relaxation.

Metrics: *FL* — test acc, train loss, rounds-to-target, acc under non-IID. *Sensing* — CRB, localization RMSE, sensing SNR, detection prob, region-uncertainty over time, tracking error. *Comm* — AirComp/aggregation MSE, uplink rate, latency, bandwidth, outage. *Energy* — sense/compute/tx energy, total energy per achieved accuracy (headline efficiency plot). *Fairness* — Jain's index (clients & regions), participation variance, region coverage fairness, age-of-sensing.

Robustness sweeps: fading type; mobility speed; K; S_{max}; non-IID severity; target/region density; sensing-noise level; imperfect CSI; power budget; dropout rate.

Result story (what the plots must show): SCOUT-FL reaches target accuracy in fewer rounds AND maintains lower region-uncertainty/CRB AND better fairness AND lower energy-per-accuracy than every baseline; the "high-SNR≠high-value" microbenchmark shows medium-SNR complementary-angle clients beating co-located high-SNR ones; ablations show each term contributes; online module tracks moving targets where offline degrades.

# I. Paper Structure (IEEE TWC outline)

Intro — ISAC+FL momentum; the perception-maintenance framing; contributions (selection problem; FIM-diversity metric; SCOUT-FL algo + guarantees; convergence link; sims).

Related Work & Positioning — explicit table vs Asaad'25 / Wen'26 / ICC-W'24 / DivFL; state the gap.

System Model — ISAC signal & channel; FIM/CRB derivation; AirComp aggregation MSE; computation/latency/energy; FL & spatial non-IID; coverage map.

Problem Formulation — unified submodular utility + constraints (MINLP-flavored).

SCOUT-FL Algorithm — submodular greedy + resource layer (+ online variant); complexity.

Theoretical Analysis — P1–P6 (P7 if online).

Simulations — setup, baselines, ablations, robustness, the microbenchmark.

Conclusion & Future Work — multi-cell/cell-free, RIS, real testbed.

# J. Risk Assessment (reviewer attacks → defense)

| Risk | Likelihood | Defense |
| --- | --- | --- |
| "Incremental over Asaad TWC-25" | High | Multi-target+coverage vs single target; FIM-diversity vs SNR/CRB-threshold; dynamic freshness; fairness; perception-maintenance framing; reproduce Asaad as a baseline and *beat it*. |
| "Sensing→convergence not new (Wen'26)" | High | Don't claim it; cite Wen; our novelty is *selection by set-dependent multi-view info + coverage*, with sensing made learning-relevant via P6. |
| "Just submodular sensor selection + FL glued together" | Med-High | P6 couples sensing data to convergence; joint constraints (CRB+MSE+energy); resource layer; online module. |
| "Greedy is trivial / weak optimization" | Med | (1−1/e) tightness, distorted-greedy for costs, comparison to convex relaxation + exhaustive; lazy-greedy complexity. |
| "Not wireless enough for TWC" | Med | Lead with ISAC signal/FIM/AirComp math; add power/bandwidth/equalizer layer; realistic fading/CSI/mobility sims. |
| "CRB objective lacks guarantee" | Med | This is *why* we optimize D-optimal log-det (M1) and treat CRB as constraint/eval (M2); state the A-optimal weak-submodularity result honestly. |
| "Synthetic-only / weak data" | Med | Add the semi-real dataset; emphasize TWC weights wireless modeling; full robustness sweeps. |
| "λ-weights hand-tuned" | Low-Med | Sensitivity analysis + a principled normalization/auto-weighting (e.g., constraint-based or Pareto-front) discussion. |
| "Submodularity breaks with correlated/realistic FIM" | Low-Med | P1 holds for any PSD J_k; show numerically on simulated correlated geometries; note A-optimal caveat. |

# K. Pre-Implementation Additions (requested — finalize before any coding)

## K.1 Final novelty statement (paper-ready, locked)

Existing FL-enabled ISAC works optimize device scheduling, beamforming, resource allocation, or sensing-aware OTA aggregation. However, they do not treat FL client selection as a set-dependent multi-view sensing information problem over a dynamic spatial environment. SCOUT-FL fills this gap by selecting clients based on marginal learning gain, log-det Fisher information gain, sensing-region freshness, spatial coverage fairness, and OTA aggregation reliability.

## K.2 Final sensing-task definition (resolves Gap #1)

Core ISAC mission = multi-target localization (per-round position/angle estimation) → clean per-client FIM → log-det submodular objective (M1) + CRB constraint/eval (M2). Main theorems (P1, P4) are stated on this version.

Dynamic extension = tracking (targets move, regions age) → posterior CRB (PCRB) + coverage/freshness map + online module (A3); carries P7.

FL learning task (what the model trains on) = wireless-sensing human activity/motion recognition (HAR) — the de-facto task across ISCC-FEEL literature (Wen, UAV-FEEL, Cao); keep MNIST/CIFAR-10 as a controllable-non-IID + Asaad-comparison track. Keep the two roles of "sensing" distinct in the writeup: localization = the *ISAC mission*; HAR = the *FL objective*.

## K.3 Mandatory microbenchmark (B) — BUILD FIRST, before any full simulation

Setup: 1 target, 3 candidate clients, budget S_{max}=2. Clients 1–2 = high sensing-SNR, nearly co-located (same viewing angle); client 3 = medium sensing-SNR, distinct/complementary angle. For each 2-subset compute accumulated FIM J(S)=J_0+Σ_{k∈S}J_k, then log-det J(S) and CRB =tr J(S)^{-1} / localization RMSE.

Expected (the paper's "aha"): {client 1, client 3} (high-SNR + complementary-angle medium-SNR) gives higher log-det and lower CRB/RMSE than {client 1, client 2} (two redundant high-SNR). Deliverable: one figure + table proving "high sensing-SNR ≠ high sensing value," motivating diversity-aware selection over any SNR-/CRB-threshold scheme. This is a gate before scaling up.

## K.4 Must-have baseline (C) — NON-NEGOTIABLE: Asaad TWC-25

Reproduce or faithfully approximate Asaad–Wang–Tabassum (TWC-25, arXiv 2501.06334). Implementation plan:

System: N-antenna PS, K single-antenna devices, one point target, Rayleigh/block-fading; CNN on MNIST + CIFAR-10 (their setup).

Selection: step-wise device dropping that omits the least-effective device per iteration via a combined global-model aggregation-MSE + single-target-CRB metric; AO downlink beamforming + uplink post-processing (equalizer).

If full reproduction is infeasible, ship a clearly-labelled "Asaad-style MSE+CRB scheduling baseline" (same combined-metric greedy drop on our simulator, stated as an approximation).

Run on the same SCOUT-FL scenarios; SCOUT-FL must win on multi-target CRB / coverage / fairness / energy-per-accuracy while matching single-target performance.

## K.5 Minimum theory floor (D) → see §G (now embedded there). Restated: (1) log-det FIM monotone submodular; (2) greedy approximation guarantee; (3) AirComp MSE affects FL convergence; (4) sensing/noise/selection affects expected learning improvement; (5) complexity. Convergence proof moderate but mandatory.

## K.6 λ weight-selection strategy (resolves Gap #7) → see §E. Constrained formulation is PRIMARY (sensing/latency/energy as hard constraints, not weighted terms); any surviving weight is normalized + sensitivity-swept with a Pareto-knee default.

## K.7 Multi-cell / cell-free extension (resolves Gap #6)

Add ONE small experiment: 2–3 coordinating ISAC BSs → multi-static sensing (richer per-target FIM from cross-BS geometry) + inter-cell AirComp interference. Same SCOUT-FL algorithm with BS-coordination added to FIM accumulation + aggregation model. Shows generalization beyond single-cell (where most competitors live); seeds the journal/future-work angle.

## K.8 Bibliography verification status (resolves Gaps #2–#3)

Confirmed this session (re-pull from IEEE Xplore before final citation): Asaad TWC-25 = 24(4):3273–3288; Wen ISCC-OTA-FEEL = Xplore doc 11143883 (TWC 2026; arXiv 2508.15185); Du IoT-J-24 = 11(21):35551–35567 (doc 10621049); ICC-W-24 = pp.804–809, doc 10615482 (Zheng, Zhu, Hu, Schmeink); Empowering-ISAC TWC-24 = doc 10768935, DOI 10.1109/TWC.2024.3502394; UAV-FEEL TWC-24 = doc 10829532, arXiv 2306.02990. TBD: Ouyang Sys-J-24 vol/issue/pages; Cao VFEEL TMC-26 doc + full author list; FedAoI(?) entry. Action: build the Related-Work table from IEEE Xplore metadata directly, not ResearchGate/secondary sources.

## K.9 Datasets & benchmarks used by the closest FL-ISAC papers (verified where marked)

✓ = confirmed from full text this session; ⚠ = abstract/IEEE-only, verify in full text before citing.

| Paper | FL task / dataset | Model | Baselines / benchmarks compared | Status |
| --- | --- | --- | --- | --- |
| Asaad TWC-25 | MNIST + CIFAR-10 (generic; sensing = echo-cancellation, *not* the data source) | CNN | existing OTA-FEEL scheduling benchmarks (names not in accessible text) | datasets ✓ / baselines ⚠ |
| Wen TWC-26 | Wireless-sensing HAR — 7 human motions (their ref [30]) | ResNet-10 (~4.9M params) | fixed-power; fixed-compute-frequency; fixed-magnitude (η=0.01); fixed-batch (250/500); OMA-based | ✓ (K=6, single-antenna, Rayleigh, M=64 subcarriers) |
| UAV-FEEL TWC-24 | Human-motion-recognition via wireless sensing | CNN | UAV-placement / resource-allocation baselines | task ✓ / details ⚠ |
| Cao VFEEL TMC-26 | Human-motion-recognition, multi-view (vertical/feature-split) | — | — | task ✓ / details ⚠ |
| Du IoT-J-24 (Fed-ISCC) | recognition (MNIST/CIFAR or HAR) | — | resource-allocation baselines | ⚠ IEEE-only |
| Ouyang Sys-J-24 | sensing classification + GAN+DQN augmentation | — | non-IID FL baselines | ⚠ IEEE-only |
| ICC-W-24 | AirComp-FL classification (likely MNIST); sensing = detection constraint | — | random / channel-aware selection + power control | ⚠ (we will reproduce) |

Takeaway: the de-facto FL task in ISCC-FEEL papers is wireless-sensing HAR; generic MNIST/CIFAR appears only where sensing is not the data source (Asaad). ⇒ SCOUT-FL primary learning task = wireless-sensing HAR; MNIST/CIFAR for controllable non-IID + head-to-head Asaad comparison.

## K.10 Candidate datasets for SCOUT-FL's own evaluation

Source: survey [Zhang et al., *IEEE COMST* 2026, Table VII] + dataset pages; verify size/license/splits from each dataset's page before use.

| Dataset | Modality | Task(s) | Size | Provider | Fit for SCOUT-FL (multi-target loc/track + spatial non-IID HAR) |
| --- | --- | --- | --- | --- | --- |
| DeepSense 6G | RGB + LiDAR + GPS + radar + mmWave | positioning, tracking, beam prediction | >1M data points | Arizona State U | PRIMARY rec. real mmWave; localization/tracking + beam; multi-viewpoint matches the diversity premise |
| WiMANS | dual-band Wi-Fi CSI + RGB video | multi-user activity recog, tracking, pose | 9.4 h, ≤5 users, 9 activities | Imperial College London | Strong secondary. multi-user + multi-view CSI → natural spatial non-IID HAR |
| OPERAnet | CSI + radar + Kinect | HAR + localization | ~8 h, 6 activities, 6 participants | Bocus et al. | multimodal HAR+loc; smaller, easy onboarding |
| Radar signatures of human activities | FMCW radar | HAR | — | Univ. of Glasgow | simple radar-HAR; low-effort baseline dataset |
| M³SC | LiDAR+RGB+depth+mmWave (simulated) | positioning, tracking, cross-modal | >1500 scenarios | Peking U | pairs with our synthetic simulator; controllable multimodal ISAC |
| MmWave Gesture / WALDO / SDP | mmWave / 802.11ay radar / CSI | gesture / indoor positioning / multi-task | varies | BUPT / NIST / IEEE-ISAC-ETI | optional task variants |
| MNIST / CIFAR-10 | images | classification | standard | — | controllable non-IID sweep + direct Asaad head-to-head |

Recommendation: synthetic simulator = primary testbed; DeepSense 6G = primary semi-real (real mmWave, localization/tracking + beam, multi-viewpoint); WiMANS = secondary (multi-user CSI HAR for spatial non-IID); MNIST/CIFAR = controllable non-IID + Asaad comparison.

# Validation & sanity checks (the "verification" step before/while executing)

Numerically verify submodularity of f_{sense},f_{cov},f_{learn} on random instances (check diminishing-returns inequality) — catches modeling bugs.

Reproduce the Asaad TWC-25 scheduling baseline (arXiv 2501.06334 has details) and confirm SCOUT-FL ≥ it on shared metrics.

Microbenchmark first: build the 2-high-SNR-co-located vs 1-medium-SNR-complementary case and confirm CRB ordering before full sims.

Empirically check P6: regress observed per-round loss decrease against selected utility U(S_t).

Ablation monotonicity: removing any term should not *improve* its own metric.

Unit/scale audit of all λ terms before tuning.

# Suggested repository / artifact layout (when execution starts)

sim/ (ISAC channel+FIM+AirComp), fl/ (FedAvg/OTA aggregation, models, non-IID partitioner), select/ (greedy/lazy/distorted, bandit/Lyapunov, baselines), theory/ (submodularity & bound checks), experiments/ (configs, sweeps), plots/, data/ (semi-real dataset loaders). One config system; seed control for reproducibility.

# Immediate next steps (post-approval)

Lock scenario parameters (K,M,R,N_t,N_r, budgets). Sensing task = multi-target localization (core) + tracking (extension); FL task = wireless-sensing HAR + MNIST/CIFAR; primary semi-real dataset = DeepSense 6G (WiMANS secondary). See §K.2, §K.9–K.10.

Screen the 4 unread arXiv ISCC-FL works + verify the FedAoI(?) entry before drafting Related Work.

Build the synthetic simulator + FIM/CRB derivation; run the microbenchmark.

Implement A1 (greedy core) + baselines; verify submodularity + reproduce Asaad.

Draft P1–P4 (easy theory) and the System Model section in parallel.

Then tackle P6 (convergence) and the A3 online module.